[SF-1449, Table of Contents, and most of Section B deleted in abbreviated version]

SECTION B

The intent of this solicitation is to obtain Weed Washing Units as defined in this solicitation, for local, Regional, and Nationwide fire suppression and all-hazard incidents.

[Section C deleted in abbreviated version]

D.1 SCOPE OF AGREEMENT

The intent of this solicitation and any resultant Agreement is to obtain Weed Washing Units for use on a local, Regional and Nationwide basis. The Contractor is responsible for all equipment, transportation to/from incident, and setup/take down necessary to meet or exceed the Agreement specifications. The resources may be used in fire suppression and all-hazard incidents. The Incident Commander or responsible Government Representative is authorized to administer the technical aspects of this agreement.

D.2 EQUIPMENT REQUIREMENTS

Equipment shall meet all standards established by specification or incorporated by reference and shall be maintained in good repair by the Contractor. The Contractor is not required to provide daily maintenance of equipment; however shall provide basic operating instructions for any equipment such as cooling units.

D.2.1 Contractor Provided Equipment - Weed Wash Containment Station Equipment

# Wash systems shall be high pressure with low volume and may be supplemented with low pressure with high volume.  High pressure systems have water pressures designated above 1000 pounds per square inch (psi), while high volume systems deliver 10 gallons of water per minute or more.

Type 1 – Self-Contained with Recycling Water System

Type 2 – Self-Contained with Non-Recycling Water System

Standard method of hire:  Daily rate which includes fully operated equipment, delivery, pickup, servicing, and mileage to/from site.

- The Host Agency will:

  - Determine weed wash needs and type of unit(s) used and area(s) of placement.

- The Government will:

  - Provide wash water to the wash site.

  - Remove waste water and solid waste.

  - Inspect washed equipment to ensure that the wash station meets agreement requirements.  If the wash station does not meet the expectations of the government, it may be removed and replaced with a different system.

  - Document contractor performance on the standard Contractor Performance System performance evaluation form.

  - Contracting Officers will make a determination of quality and performance, and non-compliance or negligence will be grounds for termination of agreement.

- The Contractor shall:

- Thoroughly wash all vehicles and equipment to remove all soil, plant parts and seeds.  Vehicles and equipment include, but are not limited to fire engines, heavy equipment, logging equipment, transports, pickups, SUVs and sedans;

- Ensure that contractor services include, but are not limited to, the removal of all mud, caked dirt, and vegetative parts off of the undercarriage, cross members, frame, skid plates, belly pans, wheels, treads, tracks, suspension, bumpers, wheel wells, radiator grills, and the ledges on the inside of rear and front bumpers;

- Visually and manually inspect hard to reach areas to ensure that they are clean;

- Inspect and wash all soil and plant parts off of drafting hoses and drafting gear on engines and water tenders;

- Ensure that the system used does not cause damage to the paint or electrical connections of vehicles and equipment being washed;

- Keep the wash station in repair and fully operational during the designated assignment;

- Capture, package and label solid waste in secure, easily transportable containment packages/devices, approved by the government representative at the incident, and place them at a location specified by the government.  Containers/packages of solid waste shall weigh no more than 50 lbs each;

- Maintain a daily record of all washed vehicles.  The contractor shall use government forms, if required by the government.

# The Contractor shall not:

# Dispose of solid waste; this is the responsibility of the government (The intention is to ensure proper disposal).

D.2.1.2 Minimum Requirements - Type 1 Self-Contained with Recycling Water System

- Portable commercial power washers with two hand-held, high pressure wands/nozzles.  These nozzles must be suitable to wash 100% of the underbody surfaces.

- Underbody washer.  The underbody washing system must have nozzles that can be directed to within 45 degrees of vertical.  The spray from these nozzles must be able to cover 100% of the underbody surfaces.

- A wash water storage tank.  The wash water storage tank shall have adequate capacity to operate the wash system continuously for a minimum of two hours.

- Waste water shall be contained by the wash system.  All wash residues shall be removed from the tracking surfaces of the vehicle being washed before vehicle exits system to prevent contamination to the exiting vehicle.

- Wash water shall be filtered to a minimum of 100 microns, or use a clean water final rinse.  Contractor is responsible for maintaining the quality of the recycled water to ensure clean and safe washed equipment. Contractor shall maintain the containment system in a functional condition at all times. Prior to disposal, all waste water shall be filtered to 100 microns or smaller particle size. Waste water shall be disposed of in accordance with wastewater requirements of the authority having jurisdiction.

- Contractor shall place solid waste in a secure, easily transportable (not to exceed 50 lbs) containment device in consultation with the ground support or resource advisor on the incident.  Solid waste shall be disposed of by the host agency.

- Process time to wash a single wildland fire engine shall not exceed 5 minutes average for any 10 fire engines.

- The contractor shall provide at least two (2) skilled operators to perform operations.  The operators shall be knowledgeable in the safe operation, maintenance, and repair of the wash system.  Operators shall be able to demonstrate knowledge, skills, and abilities to manage all waste products from the washer system.  These personnel shall be present at all times during the incident operational periods, and are responsible for the safe operation of the wash station.

- Two, 1000 watt halogen work lights on stands and GFI module.

- The wash system must comply with all applicable OSHA regulations related to operator safety and all segments of the washer must be in operating condition with no missing parts.  All alternating current electric motors shall be listed with Underwriters Laboratory.

D.2.1.2 Minimum Requirements - Type 2:  Self-Contained with Non-Recycling Water System (this may be a direct-draining or remote discharge system)

- Portable commercial power washer with two hand-held, high pressure wand/nozzles.  These nozzles must be suitable to wash 100% of the underbody surfaces.

- A wash water storage tank.  The wash water storage tank shall have adequate capacity to operate the wash system continuously for a minimum of two hours.

- Waste water shall be disposed of in accordance with wastewater requirements of the authority having jurisdiction.

- Remote discharge systems shall have an adequate means to pump all waste water at least 200’ from the wash station.  Prior to disposal, all waste water shall be filtered to a minimum of 100 microns or smaller particle size, or through dewatering bags fabricated from Amoco 4553 or equivalent geotextile cloth, having a maximum apparent opening size of 150 microns.

- Direct draining systems shall utilize Amoco 4553 or equivalent geotextile cloth, having a maximum apparent opening size of 150 microns.  This cloth shall withstand heavy truck traffic.  A 15’ wide by 40’ piece shall be supplied by the contractor and shall be installed on a pad of gravel or a well-drained surface that is provided by the host agency.  Contractor shall maintain the mat in a functional condition at all times.  All solid waste greater than 150 micron size, including all geotextile cloth pieces, shall be placed in a secure, easily transportable containment (not to exceed 50 lbs) device in consultation with the ground support or resource advisor on the incident.  Solid waste shall be disposed of by the host agency.

- Process time to wash a single wildland fire engine under normal conditions shall not exceed 8 minutes average for any 10 engines.

- The contractor shall provide at least two (2) skilled operators to perform operations.  The operators shall be knowledgeable in the safe operation, maintenance, and repair of the wash system.  Operators shall be able to demonstrate knowledge, skills, and abilities to manage all waste products from the washer system.  These personnel shall be present at all times during the incident operational periods, and are responsible for the safe operation of the wash station.

- The wash system must comply with all applicable OSHA regulations related to operator safety and all segments of the washer must be in operating condition with no missing parts.  All alternating current electric motors shall be listed with Underwriters Laboratory.

- Two (2), 1000 watt halogen work lights on stands and GFI module.

- The system may have:

    - A mechanical underbody washer.  The underbody washing system must have nozzles that can be directed to within 45 degrees of vertical.  The spray from these nozzles must be able to cover 100% of the underbody surfaces.

    - An additional operator, skilled and knowledgeable in the safe operation, maintenance and repair of the wash system. Operators shall be able to demonstrate knowledge, skills and abilities to manage all waste products from the washer system. These personnel shall be present at all times during the incident operational periods, and are responsible for the safe operation of the wash station.

D.3 Not Applicable

D.4 EQUIPMENT RELIABILITY

The Contractor shall provide dependable equipment that meets all applicable state and federal laws relating to motor vehicles and equipment.

The Government reserves the right to conduct inspections at any time.

[D.5 through D.6.1 deleted in abbreviated version]

D.6.2 RANKING OF AWARDED RESOURCES FOR DISPATCH PRIORITY

All resources on an awarded Agreement will be ranked on a dispatch priority list by Host Dispatch Zone or Geographic Area.  Priority will be given to those small business concerns identified in Section B, Method of Award - Cascading Set-Aside Procedure. Within each small business program category, priority will be given according to the price offered for the type of resource. The following calculation will be used to determine the lowest price, with the lowest price being ranked highest on the dispatch list.

[D.6.3 through D.21.7 deleted in abbreviated version]

D.21.8 PAYMENTS

The host agency for each incident is responsible for payments. The payment office will be designated in block 9 on the Emergency Equipment - Use Invoice, Form OF-286. See Exhibit B for complete agency payment office information.

The time under hire shall start at the time the resource begins traveling to the incident after being ordered by the Government, and end at the estimated time of arrival back to the point of hire after being released, except as provided in D.21.8.3.

- On-Shift includes time worked, time that resource is held or directed to be in a state of readiness, and compensable travel (resource traveling under its own power) that has a specific start and ending time.

- Daily Rate - Payment will be made on basis of calendar days (0001 - 2400). For fractional days at the beginning and ending of time under hire, payment will be based on 50 percent of the Daily Rate for periods less than 8 hours.

[D.21.9 through D.23 and all exhibits deleted in abbreviated version]