INFRA Definitions Directions

| R - RECONTOURED | The road prism has been fully obliterated. (top of cut to toe of fill) by restoring natural contours and slopes. All CMP’s have been removed and all watercourses have been restored to natural channels and floodplains. |
| --- | --- |
| B – BLOCKED | Entrance has been blocked. No other treatment to road prism or drainage. |
| C – CMP REMOVED | CMP’s on streams and drainages have been removed and associated fills have been sloped back and stabilized. Ditch relief culverts may be bypassed and left in place or removed. Former drainage patterns have been re-established. |
| V – REVEGETATED | Road prism has been re-vegetated. |
| U – UNSTABLE FILLS REMOVED | Unstable fills have been removed. |
| W – WATERBARRED/OUTSLOPED | The road prism has been water barred and/or outsloped. |
| R - RECONTOURED | The road prism has been fully obliterated. (top of cut to toe of fill) by restoring natural contours and slopes. All CMP’s have been removed and all watercourses have been restored to natural channels and floodplains. |
| CU – COMBINATION | CMP’s have been removed. Unstable fills have been removed. |
| CW – COMBINATION | CMP’s have been removed The road prism has been water barred and/or outsloped. |
| CV - COMBINATION | CMP’s have been removed. The road prism has been re-vegetated |
| VU - COMBINATION | The road prism has been re-vegetated and unstable fills have been removed. |
| WV - COMBINATION | The road prism has been water barred and/or outsloped, and re-vegetated. |
| WU - COMBINATION | The road prism has been water barred and/or outsloped. Unstable fills have been removed. |
| CWU - COMBINATION | CMP’s have been removed The road prism has been water barred and/or outsloped, and unstable fills have been removed. |
| CWV - COMBINATION | CMP’s have been removed. The road prism has been water barred and/or outsloped, and re-vegetated. |
| CVU - COMBINATION | CMP’s have been removed. The road prism has been re-vegetated. Unstable fills have been removed. |
| WVU - COMBINATION | The road prism has been water barred and/or outsloped and re-vegetated. Unstable fills have been removed. |
| WUVC - COMBINATION | CMP’s have been removed. The road prism has been re-vegetated, water barred and/or outsloped. Unstable fills have been removed. |

| GPS – GLOBAL POSITIONING | Mileage determined on the ground using Global Positioning equipment. |
| --- | --- |
| MAPW – MAP WHEEL | Mileage determined by using a map wheel from existing maps. |
| WHEEL – MEASURING WHEEL | Mileage determined on the ground using a measuring wheel |
| ODOM – ODOMETER READING | Mileage determined on the ground by vehicle odometer reading |
| OTHER – OTHER SOURCE | Other mileage source – specify in remarks such as cloth tape, steel chain, etc. |
| STRING – STRING BOX | Mileage determined on the ground using a string box (hip chain) |
| SURVEY – CONSTRUCTION SURVEY | Mileage based on survey done for original construction or reconstruction. |
| PED – PEDOMETER | Mileage determine by walking with a pedometer. |

| A - DITCH, SHOULDER, AND BERM | Shoulder and Berm |
| --- | --- |
| B - DITCH AND SHOULDER | Ditch and shoulder |
| C - DITCH AND BERM | Ditch and berm |
| D - DITCH | Ditch |
| E - SHOULDER | Shoulder |
| F - SHOULDER AND BERM | Shoulder and berm |
| G - BERM | Berm |
| H - NO DITCH, SHOULDER, OR BERM | No ditch shoulder or berm |
| P - PRIMITIVE | Primitive, not constructed to a standard |

|  | User Requirements for Population |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Road System |  | Requirement |  |  |  |  |  |  |  |
|  | ALL |  | Required |  |  |  |  |  |  |  |
| Business Rules: | - Decommissioning eliminates all maintenance needs for the asset. (Fed Acct Stds Advisory Board/ 1998). It results in the stabilization and restoration of unneeded roads to a more natural state (36 CFR 212.1) - Road Management decisions must be informed by travel analysis and disclosed in an appropriate NEPA Document (FSM 7712.11). Only change the route status through an appropriate decision document. - Only change the Route Status after the appropriate NEPA decision has been made and the work (if required) has been completed. - Use this linear event to document decommissioning. Don’t delete the route from the database and road feature class. - Temporary roads are a local decision to either remove or preserve the data within Infra. - This linear event is not extensible by the forests. |  |  |  |  |  |  |  |  |  |

| VALID CODES - ROUTE STATUS |  |  |  |
| --- | --- | --- | --- |
| CODE | DESCRIPTION |  |  |
| CV – CONVERTED | A route that was no longer needed and has been converted to another use |  |  |
| DE - DECOMMISSIONED | A route that was no longer needed and has been removed from service. |  |  |
| EX - EXISTING | A route that physically exists. |  |  |
| PL - PLANNED | Planned route identified in a completed NEPA document, with a Record of Decision. |  |  |
| VALID CODES - SURFACE TYPE |  |  |
| CODE |  | DESCRIPTION |
| AC - ASPHALT |  | Asphaltic Concrete |
| AGG - CRUSHED AGGREGATE OR GRAVEL |  | Crushed or screened graded material |
| BST - BITUMINOUS SURFACE TREATMENT |  | Built up surface of asphalt emulsion and aggregate, not a dust palliative. |
| CSOIL - COMPACTED SOIL |  | Compacted Native Material |
| FSOIL – FROZEN SOIL |  | Template has been cleared and rough shape completed but can’t be used until frozen conditions exist. |
| IMP - IMPROVED NATIVE MATERIAL |  | Imported or processed material (pit run, select borrow, or admixture added to the surface soil and compacted) |
| NAT - NATIVE MATERIAL |  | No imported or processed materials |
| P - PAVED |  | Unknown manufactured hard surface |
| PCC – PORTLAND CEMENT CONCRETE |  | Portland Cement Concrete |

Decommission.  Demolition, dismantling, removal, obliteration and/or disposal of a deteriorated or otherwise unneeded asset or component, including necessary cleanup work. This action eliminates the deferred maintenance needs for the fixed asset. Portions of an asset or component may remain if they do not cause problems nor require maintenance.  (Financial Health - Common Definitions for Maintenance and Construction Terms, July 22, 1998)

Deferred Maintenance.  Maintenance that was not performed when it should have been or when it was scheduled and which, therefore, was put off or delayed for a future period. When allowed to accumulate without limits or consideration of useful life, deferred maintenance leads to deterioration of performance, increased costs to repair, and decrease in asset value. Deferred maintenance needs may be categorized as critical or non-critical at any point in time. Continued deferral of non-critical maintenance will normally result in an increase in critical deferred maintenance. Code compliance (e.g. life safety, ADA, OSHA, environmental, etc.), Forest Plan Direction, Best Management Practices, Biological Evaluations, other regulatory or Executive Order compliance requirements, or applicable standards not met on schedule are considered deferred maintenance.  (Financial Health - Common Definitions for Maintenance and Construction Terms, July 22, 1998)