BATCH MIXER INSPECTION CHECKLIST

# Company Name:  _________________________________________________

# Batch Mixer Identification:  ________________________________________

Inspection Location:  ______________________________________________

Date:  _________________________

Inspector:  _____________________

# Trailer Mounting

|  | Tank Connected to Trailer Frame Not Expanded Metal Decking |
| --- | --- |
|  | Trailer Equipped With Brakes if Trailer Rating is 1500 lbs or More |
|  | Trailer Wiring Protected from Abrasion |

Comments:

# Engine Installation

|  | Fuel Tank Located to Reduce Spillage of Gasoline on Hot Engine |
| --- | --- |
|  | Shielding Installed Between Pump and Engine to Prevent Leaks From Contacting Hot Engine |
|  | Shielding Installed Between Piping and Engine to Prevent Leaks From Contacting Hot Engine |

Comments:
BATCH MIXER INSPECTION CHECKLIST (Continued)

# Tank (Greater Than 119 Gallons)

|  | MC 306 or DOT 406 Specification Cargo Tank or IBC (Type_____________________________) |
| --- | --- |
|  | Emergency Shutoff Lever Accessible and Remotely Actuated More Than 10 Feet Away From Shutoff Valve or At End of Tank Furthest From Valve |
|  | Tank and/or Vehicle Placarded on 4 Sides as “Flammable Liquid” and Marked on 4 Sides as “UN 1203” |
|  | Shut Off Valves are Protected and Do Not Protrude from Vehicle |
|  | 2” Cam & Groove Fitting Installed for Vapor Removal/Recovery (Vapor Removal/Recovery hoses minimum of 50 foot sections) |
|  | Fill Spout Extends to Within 6” of Tank Bottom – Splash Filling Not Permitted |
|  | V, K Inspection Current |
|  | I, P Inspection Current |

Comments:

# Electrical

|  | All Electrical Connections Are Sealed and Secured |
| --- | --- |
|  | Switch Housings Covered |
|  | Battery Located Away From Piping Joints and Pump |

Comments:

BATCH MIXER INSPECTION CHECKLIST (Continued)

# Hoses

|  | Hoses Designed for Use With Gasoline (Hose Make & Model ____________________________) |
| --- | --- |
|  | Vapor Recovery/Removal Hose Designed for Use With Gasoline Vapor (Vapor Removal/Recovery hoses minimum of 50 foot sections) (Hose Make & Model ____________________________________________________________) |
|  | Swaged Hose Ends |
|  | Electrically Conductive |
|  | Live Reel Installed (Minimum of 60 foot hose) |

Comments:

# Pump

|  | Pump Internals are Non-Sparking (Pump Make & Model_______________________________) |
| --- | --- |
|  | Pump Seals are Compatible With Gasoline (Viton or Buna N) |

Comments:

# Misc

|  | Safety Pins Installed on Camlok Fittings or Self Locking Camlok Fittings Installed |
| --- | --- |
|  | Pressure Gauge Isolated From Gel |
|  | Valves Labeled As To Function and Flow Direction |
|  | Fire Extinguisher Inspected, Mounted and Accessible |

Comments: