HELITORCH INSPECTION CHECKLIST

Company Name:  _________________________________________________

Helitorch Identification:  ________________________________________

Inspection Location:  ______________________________________________

Date:  _________________________

Inspector:  _____________________

Tank (55 to 70 Gallons)

[image: image1.jpg])

%_1280.00_Piiet)

o
4
£
]
a
a
8
&)

t Projects)Aes's\SHEET

Ci\patatCu

1:0.00608026

VAPOR REMOVAL
CONNECTION (PT COUPLING
2”7 vSL2 DUST CAP AND 27
MALE ADAPTER X MALE
THREAD). WHEN THE VAFOR
HOSE IS NOT INSTALLED
THIS CONNECTION SHOULD
BE CAPPED.

RELIEF VALVE CONNECTION
(CLAY AND BAILEY MODEL 37X,
PART NUMBER 0037-03-2030,
PT COUPLING 2" DSL2 FEMALE

COUPLER X FEMALE THREAD,

AND PT COUPLING 27 MALE
ADAPTER X MALE THREAD.

SIGHT GLASS (JOHN C
ERNST 2” MODEL 322)
PROVIDES LIGHT FOR

VIEWING.

SIGHT GLASS (JOHN C

- ERNST 2” MODEL 322)

TO DETERMINE FUEL
e LEVEL.

SIGHT GLASS (JOHN C
ERNST 2" MODEL 322) PUMP

TO DETERMINE FUEL
LEVEL.

FILL CONNECTION
(EMCO WHEATON
J72A—AVNO—A).

SIMPLEX 5400 AND FIRE SPEC 2000
HELITORCH CONFIGURATION

NOTE: RELIEF VALVE INSTALLATION IS REQUIRED





HELITORCH INSPECTION CHECKLIST (Continued)
	
	DOT Specification Drum (UN1A1 or UN1A2) or Exemption for Non-DOT Fuel Tank

(Drum Spec or Exemption Number__________________________________________________)

	
	

	
	Flammable Liquid” Label, “UN 1203” Marking, “Gasoline” Marking and Exemption                               Number (As Required) Applied to Drum or Tank

	
	

	
	2” Male Cam & Groove Fitting Installed for Vapor Removal/Recovery (Per Sketch for Fire Spec & Simplex, Vapor Removal/Recovery hoses minimum of 50 foot sections)

	
	

	
	Relief Valve Installed and Mounted on Cam and Groove Fittings to Prevent Clogging of Valve by Gel  (Per Sketch for Fire Spec & Simplex)

	
	

	
	Cam and Groove Fitting Levers Secured With Safety Pins or Self Locking Levers Are Installed

	
	

	
	2” Emco Wheaton Dry Break Adapter Installed for Fueling of Drum or Tank (Per Sketch for Fire Spec & Simplex)

	
	

	
	Filling of Drum of Tank by Bottom Filling – Either by Installation of Dry Break at Bottom of Drum or by a Fill Spout That Extends to Within 6” of Tank Bottom – Splash Filling Not Permitted

	
	

	
	Sight Glasses Installed to Determine Fuel Level in Drum or Tank

	
	

	
	Drum or Tank is Not Damaged and No Leakage is Visually Detectable


Comments:

HELITORCH INSPECTION CHECKLIST (Continued)
Suspension
	
	Wire Ropes Have No Physical Damage (Broken Strands, Kinks, Etc)

	
	

	
	Aviation Grade Bolts Installed – The Bolt Shoulders are Long Enough So That the Cable Ends Contact Only The Shoulder of the Bolts and the Bolt Threads are not a Load Bearing Surface

	
	

	
	The Pear Link Adapter Spacer is Installed so that the Pear Link Contacts Only the Spacer and Not the Bolt


Comments:
Electrical

	
	Power Cable in Good Condition – No Cuts or Gaps in Insulation


Comments:

Misc.
	
	Propane Hose - Hose is Compatible With Propane and Has a Braided Metal Cover


Comments:
Page 1 of 3

