Helitorch Inspection Checklist

| Project/Incident |  |  |  | Location |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Helitorch Manager |  |  |  |  |  |  | Date |  |
| Helitorch Mixmaster |  |  |  |  |  |  | Date |  |
| ## DAILY INSPECTION |  |  |  |  |  |  |  |  |
| ### HELITORCH DRUM/TANK |  |  |  |  |  |  |  |  |
|  |  | Visually inspect for damage and leaks |  |  |  |  |  |  |
|  |  | Clean and ready for use |  |  |  |  |  |  |
|  |  | Valves clean and working properly |  |  |  |  |  |  |
|  |  | All fittings in place and leak free |  |  |  |  |  |  |
|  |  | Removable drum head in place, securing band tight, and leak free |  |  |  |  |  |  |
|  |  | All bolts/pins in place and secure |  |  |  |  |  |  |
|  |  | Bonding cable connection paint free and clean to allow continuity |  |  |  |  |  |  |
|  |  | Replacement drum available on-site |  |  |  |  |  |  |
| ### HELITORCH FRAME |  |  |  |  |  |  |  |  |
|  |  | No cracks or breaks |  |  |  |  |  |  |
|  |  | Clean and ready for use |  |  |  |  |  |  |
|  |  | All bolts/pins in place and secure |  |  |  |  |  |  |
| ### MOTOR AND ELECTRICAL HOUSING |  |  |  |  |  |  |  |  |
|  |  | Clean |  |  |  |  |  |  |
|  |  | Motor pulley and belt in good condition |  |  |  |  |  |  |
|  |  | Electrical wiring free from cracks, corrosion and connected properly |  |  |  |  |  |  |
|  |  | Pump motor lubricated |  |  |  |  |  |  |
|  |  | All screws and bolts in place and tight |  |  |  |  |  |  |
|  |  | Pump operating (primed) |  |  |  |  |  |  |
|  |  | Ignition system adjusted, clean and working |  |  |  |  |  |  |
| ### HOSE CONNECTIONS |  |  |  |  |  |  |  |  |
|  |  | Clean with clamp or compression fittings tight/leak free |  |  |  |  |  |  |
|  |  | Dry Break Valves clean. Caution! Do not open valve unless attached! |  |  |  |  |  |  |
|  |  | Swivel rotates freely |  |  |  |  |  |  |
| ### SUSPENSION SYSTEM AND HELITORCH ELECTRICAL CABLE |  |  |  |  |  |  |  |  |
|  |  | Clean and free of kinks, nicks, corrosion, and burrs |  |  |  |  |  |  |
|  |  | Suspension line connectors secure and in good condition |  |  |  |  |  |  |
|  |  | All bolts, nuts, and attachment ring meet MTDC drawing, properly secured |  |  |  |  |  |  |
|  |  | Electrical connector clean and tight w/line properly attached to suspension system and Helitorch |  |  |  |  |  |  |
|  |  | Separator bars not cracked or broken and properly attached between cable swedges |  |  |  |  |  |  |
| HELITORCH SUPPORT KIT |  |  |  |  |  |  |  |  |
|  | ### Windsock |  |  |  |  | ### Dust mask |  |  |
|  | Approved powder dispenser |  |  |  |  | First aid kit/burn kit |  |  |
|  | Two 5-gallon slop buckets |  |  |  |  | Chemical splash goggles |  |  |
|  | Continuity tester |  |  |  |  | Spare dry break valves |  |  |
|  | Fire extinguishers, one 40-B:C per aircraft, minimum of two-3 gallon class B compressed air foam, or class B capable engine |  |  |  |  | ### 100 % Cotton coveralls or Fire Retardant clothing labeled non-static (variety of sizes) |  |  |
|  | Tool kit |  |  |  |  | Wire brush, steel wool |  |  |
|  | Spare parts kit |  |  |  |  | Wire ties |  |  |
|  | Hearing protection |  |  |  |  | Electrical & duct tape |  |  |
|  | Orange paint/fluorescent flagging/pad markers |  |  |  |  | Silicon based lubricant/engine de-greaser |  |  |
|  | Bonding cables |  |  |  |  | Washbasin, soap, and 5 gallons of water |  |  |
|  | Emery cloth and extra tip parts |  |  |  |  | Hand cleaner and rags |  |  |
|  | Metal funnel and coffee can |  |  |  |  | Scale and scoop for measuring gelling agent |  |  |
|  | Flight helmet, flight suit |  |  |  |  | Fuel thermometer |  |  |
|  | Approved vapor recovery/removal hose 2” |  |  |  |  | Clay Bailey pressure relief valve |  |  |
|  | 2 extra sight glasses |  |  |  |  | Nonferrous paddle/scraper |  |  |
|  | 5 gallons of diesel for cleanup |  |  |  |  | Cleaning rags, hand cleaner, garbage bags |  |  |
|  | 2 extra drum head seals |  |  |  |  | Nitrile, cotton, leather gloves |  |  |
|  | Grease gun w/grease |  |  |  |  | Single pole guarded electrical switch |  |  |
|  | Extra nut and bolts |  |  |  |  | Propane bottles if needed |  |  |
|  | 5-gallon hazmat spill kit |  |  |  |  | Headset/patch cords |  |  |
|  | 2 nonferrous metal pipe wrenches |  |  |  |  | Eyewash Station |  |  |
| Helitorch Mixmaster |  | Signature: |  |  |  |  |  | Date |