# MIX-TRANSFER SYSTEM INSPECTION CHECKLIST

# Company Name:  _________________________________________________

# System Identification:  ________________________________________

Inspection Location:  ______________________________________________

Date:  _________________________

Inspector:  _____________________

# Drums

# MIX-TRANSFER SYSTEM INSPECTION CHECKLIST (Continued)

|  | DOT Specification Drums |
| --- | --- |
|  | “Flammable Liquid” Label, “UN 1203” Marking, “Gasoline” marking and Exemption Number (As Required) Applied to Each Drum |
|  | 2” Male Cam & Groove Fitting Installed for Vapor Removal/Recovery (Per Sketch, Vapor Removal/Recovery hoses minimum of 50 foot sections) |
|  | 2” Male Cam & Groove Fitting Installed for Gelling Agent Dispenser and Gasoline Fill Port (Per Sketch) |
|  | Relief Valve Installed and Mounted on Cam and Groove Fittings to Prevent Clogging of Valve by Gel (Per Sketch) |
|  | Cam and Groove Fitting Levers Secured With Safety Pins or Self Locking Levers Installed |
|  | 2” Emco Wheaton Dry Break Adapter Installed for Pump Discharge and Pump Suction Connections (Per Sketch) |
|  | Bonding Lugs Installed on Pump Discharge Dry Breaks |
|  | Sight Glass Installed to Determine Fuel Level in Drum (Per Sketch) |
|  | Drum is Not Damaged and No Leakage is Visually Detectable |

Comments:

# Engine Installation

|  | Shielding Installed Between Pump and Engine to Prevent Leaks From Contacting Hot Engine |
| --- | --- |

Comments:

# MIX-TRANSFER SYSTEM INSPECTION CHECKLIST (Continued)

# Hoses

|  | Hoses Designed for Use With Gasoline (Hose Make & Model ____________________________) |
| --- | --- |
|  | Vapor Recovery/Removal Hose Designed for Use With Gasoline |
|  | Vapor (Vapor Removal/Recovery hoses minimum of 50 foot sections) (Hose Make & Model ______________________________________________________) |
|  | Swaged Hose Ends |
|  | Electrically Conductive |

Comments:

# Pump

|  | Pump Internals are Non-Sparking (Pump Make and Model _______________________________) |
| --- | --- |
|  | Pump Seals are Compatible With Gasoline (Viton or Buna N) |

Comments:

# Misc

|  | Pressure Gauge Isolated From Gel |
| --- | --- |
|  | Valves Labeled As To Function and Flow Direction |

Comments: