#### PSD Air Operations/Safety GO/NO GO Checklist

#### The helicopter operations on this project require the use of this checklist.  If all items are not checked as satisfactory and maintained in that state for the duration of the mission, flying operations will be suspended until the deficiency is mitigated.

Helibase Safety 		Approved Project Aviation Safety Plan.

Qualified Helibase/Helispot Manager assigned.

Helibase/helispot meet established standards.

Organizational chart posted, assignments known.

Communications chart posted.  Frequency assignments known.

Helibase/helispot fire protection meets established standards.

Crash rescue/evacuation kits on the helibase/helispot.

Current Aviation Incident Response Plan posted at Dispatch/Helibase and ready to implement.

All personnel briefed.  Aerial ignition personnel briefed on in-flight operations.

Separation of aircraft (if more than one used).

Personal protective equipment meet established standards.

Flight hazard map posted/hazards known to pilot.

Aircraft/Pilot(s)		Check pilot and aircraft approval cards.

Check pilot and aircraft limitations.

Load calculations prepared and posted.

Check aircraft radios.

Remove all loose articles from aircraft.

Fire shelter on board aircraft for each person.

Water bucket ordered with aircraft (optional).

Approved Secondary Restraint Harness and approved aircraft hardpoint to attach to.

Plastic Sphere		Installation correct with restraints in place.

Dispenser 		Mechanical operation satisfactory.

Extinguisher (water reservoir) system filled and operational.

Glycol reservoir filled and tightly capped.

20-second ignition delay achieved.

Intercom and aircraft-to-ground communications operable.

Pilot has been briefed and agrees that all is in order.

Sphere containers secured.

Seat belt cutter available for emergency use.

Additional container of water available.

Tool kit/Premo Mark III manual on board aircraft (optional).

Burning		All persons briefed and assignments known.

Operations 		Maps/photos of project area used/posted.

Special weather considerations known/discussed.

Communication plan posted and frequency assignments known.

Emergency operations plan known and discussed.

Personal protective equipment meets established standards.

Special safety considerations known and discussed.

Support		Adequate support equipment/personnel to complete mission.

Equipment/ 		Pump/engine operational checks.

Personnel 		Radios/communications operationally checked.

Support equipment/personnel propositioned before actual operations begin.

Adequate supply of plastic spheres and glycol to complete project.

PSD checklist complete.

/	  		/

PSD Operator	Date 		Pilot	Date

/

Burn Boss/Ignition Specialist    Date