Briefing Paper

Stinger-Steered Chip Trailer

USFS San Dimas Technology and Development Center

Project leader:  Dave Haston

Technician/Designer:  Joe Fleming

Contact info:

dhaston@fs.fed.us

jdfleming@fs.fed.us

SDTDC:  (909)599-1267

Need:  Timber sales are increasingly being sold with a fuels reduction component that often includes chipping.  Chip vans are typically used to transport material from the sale area.  Unfortunately, most forest roads were designed to accommodate traditional west coast-style stinger-steered log trucks, and were not designed with chip trailers in mind.  Conventional chip trailers have significant off-tracking problems (increasing with trailer length) as well as decreased ground clearance.

Concept:

- Convert conventional stinger-steered log trucks for chip hauling

- Utilize standard ISO shipping containers

- Separate sub frames (front and rear) to create stinger/reach clearance for rolling dips and curves, as well as allowing the use of various container lengths

- Containers available in different lengths (20/40/45/48/53’) / heights (8.5 and 9.5’), closed and open top

Estimated Cost (assume tractor and stinger trailer available):

Basic system (closed top, use dump table)		Advanced system

Container		$2,500				Open-top container		$2,500

Blow-in chip door	$3,000				Side-to-side roll tarp		$2,000

Sub frames		$5,000				Walking floor			$24,000

Landing gear		$2,000				Wet kit (PTO/pump)		$2,500

Misc. *		$1,500				Sub total			$31,000

$14,000			Total, as-built		         ~	$45,000

* Used 5th wheel, new cups (saucer), plywood, etc.

Desirable tractor features:

Air ride suspension (for deploying landing gear)

Quick-change system (to interchange log bunk and fifth wheel)

Wet kit (to power live floor)

Potential issues:

Disconnecting trailer on dump tables that do not accommodate tractor.

Structural integrity and long term durability of container (unsupported mid-span)

Operating parameters:

Container Volume: 88 cubic yards

Estimated payload:  21-22 tons @80,000 lb

Height:  13’ 3” (50-state legal)