SECTION 00250

CONSTRUCTION STAKING

PART 1 - GENERAL

## 1.1	SUMMARY

#### This work shall consist of the construction staking of a rappel tower in accordance with the drawings and specifications.  The work includes the furnishing of all labor, equipment, instruments, materials, transportation and other incidentals necessary to complete the construction staking in accordance with these specifications and acceptable engineering practice.

#### Construction staking shall be accomplished under the direction of a Civil Engineer, Engineering Technician, or Land Surveyor closely associated and familiar with construction staking; periodic visits to the project site are required.

#### Measurement shall be lump sum for surveying and staking the project through all phases of construction.

PART 2 – PRODUCTS

## GOVERNMENT-FURNISHED SURVEY CONTROL SHEETS

#### The Government will provide the contractor with a set of survey control sheets showing horizontal and vertical control for the rappel tower.

## Stakes

#### Identification stakes and hubs shall be of sufficient length and width to provide and solid set in the ground and to provide space for marking above ground when applicable.  Other dimensions and materials may be used, such as steel reinforcing bars, wire flagging and markers, and metal pins, if approved in writing by the Contracting Officer (CO).

## SURVEY NOTE PAPER AND BOOKS

#### Paper for survey notes shall be moisture-resistant paper.  Notes shall be contained in books with covers that will protect the contents and retain the pages in numerical sequence during field use.  Field notebooks or note paper shall be furnished by the contractor.

PART 3 – EXECUTION

## GENERAL

#### The Contractor shall be required to provide all necessary staking and control for all phases of construction.  The contractor shall use the data and information shown on the project drawings to establish the location of all facilities to be constructed under this contract.  The Contractor may choose any method of establishing horizontal and vertical controls provided the following minimum precision and accuracy requirements of .01 feet are met..

## Discrepancies

#### The contractor shall compare the staked horizontal and vertical alignment with the design data.  Differences between previously recorded and observed elevations of bench marks shall be referred to the Contracting Officer.  Staking of these areas shall be deferred until the Contacting Officer resolves these differences.

## Vertical Control

#### Vertical control for construction shall be established by the Forest Service prior to start of construction.

## Survey Notes

#### All survey data shall be neatly recorded in survey books.  All survey notes shall become the property of the Forest Service.  Errors shall be deleted by lining out.  Date, crew names and positions, instrumentation, and weather shall be recorded in the notes at the beginning of each day’s work.

END OF SECTION  00250