SECTION 01430

- CONTRACTOR QUALITY CONTROL

PART 1	GENERAL

## DESCRIPTION

#### The work of this section consists of preparing and executing a quality control program.

## SUBMITTALS

#### As specified in Division 1 Section “Submittal Procedure”.

#### Quality Control Plan:

##### At the time of the preconstruction conference, submit for approval a written Contractor Quality Control (CQC) plan.

##### If the plan requires any revisions or corrections, the Contractor shall resubmit the plan within 10 days.

##### The Government reserves the right to require changes in the plan during the contract period as necessary to obtain the quality specified.

##### No change in the approved plan may be made without written concurrence by the Contracting Officer.

##### The plan shall include:

###### A list of personnel responsible for quality control and assigned duties.  Include each person's qualifications.

###### A copy of a letter of direction to the Contractor's Quality Control Supervisor outlining assigned duties.

###### Names, qualifications, and descriptions of laboratories to perform sampling and testing, and samples of proposed report forms.

###### Methods of performing, documenting, and enforcing quality control of all work.

###### Methods of monitoring and controlling environmental pollution and contamination as required by regulations and laws.

#### Contractor's Quality Control Daily Reports:  Submit showing all inspections and tests on the first workday following the date covered by the report.

#### Test Reports:

##### Submit Daily Test Information Sheets with Quality Control Daily Reports.

##### Submit failing test results and proposed remedial actions within four hours of noted deficiency.

##### Submit three copies of complete test results not later than three calendar days after the test was performed.

#### Off-Site Inspection Reports:  Submit prior to shipment.

#### If the CQC plan and Quality Control Daily Reports are not submitted as specified, the Contracting Officer may retain all payments until such time a plan is approved and implemented.

## QUALITY ASSURANCE

#### General:

##### The quality of all work shall be the responsibility of the Contractor.  Testing shall be the responsibility of an independent testing laboratory

##### Inspect and test all work often enough to ensure that the quality of materials, workmanship, construction, finish, and functional performance is in compliance with applicable specifications and drawings.

##### Quality Control Daily Reports shall be completed by the Quality Control Supervisor.

##### Test reports shall be completed by person performing the test.

##### The Contracting Officer may designate locations of tests.

            - Design and complexity of the project.

            - Location of the project.

            - Characteristics of area construction labor market.

            - Duration of project.

            - Cost and type of contract.

            - Amount and type of off-site fabrication.

#### Quality Control Staff:

##### The Contractor’s Quality Control Supervisor may also perform the duties of Project Superintendent.

##### The Contractor’s Quality Control Supervisor shall be assigned no other duties.

##### The Contractor’s designated Quality Control Supervisor shall be on the project site whenever contract work is in progress.

##### The Contractor’s job supervisory staff may be used to assist the Quality Control Supervisor, supplemented as necessary by additional certified testing technicians.

#### Testing Laboratory and Equipment:

##### Employ certified independent laboratories to perform sampling and testing.  The testing laboratory organization shall be certified for the type of testing work to be done.

##### All measuring devices, laboratory equipment, and instruments shall be calibrated at established intervals against certified standards in accordance with NBS requirements.  Upon request, measuring and testing devices shall be made available for use by the Government for verification tests.

PART 2	PRODUCTS		Not used.

PART 3	EXECUTION

## OFF-SITE CONTROL

#### Items that are fabricated or assembled off-site shall be inspected for quality control at the place of fabrication.

## ON-SITE CONTROL

#### Notification:

##### Notify the Contracting Officer at least 48 hours in advance of the preparatory phase meeting.

##### Notify the Contracting Officer at least 24 hours in advance of the initial and follow-up phases.

#### Preparatory Phase:  Perform before beginning each feature of work.

##### Review control submittal requirements with personnel directly responsible for the quality control work.  As a minimum, the Contractor's Quality Control Supervisor and the foreman responsible for the feature of work shall be in attendance.

##### Review all applicable specifications sections and drawings related to the feature of work.

##### Ensure that copies of all referenced standards related to sampling, testing, and execution for the feature of work are available on site.

##### Ensure that provisions have been made for field control testing.

##### Examine the work area to ensure that all preliminary work has been completed.

##### Verify all field dimensions and advise the Contracting Officer of discrepancies with contract documents.

##### Ensure that necessary equipment and materials are at the project site and that they comply with approved shop drawings and submittals.

##### Prepare a report on all preparatory phase activities and discussions.  Attach report to Contractor's Quality Control Daily Report.

#### Initial Phase:

##### As soon as work begins, inspect and test a representative portion of a particular feature of work for quality of workmanship.

##### Review control-testing procedures to ensure compliance with contract requirements.

##### Prepare a report on all initial phase activities and discussions.  Attach report to Contractor's Quality Control Daily Report.  Exact location of initial phase shall be indicated for future reference and comparison with follow-up phases.

#### Follow-Up Phase:  Inspect and test as work progresses to ensure compliance with contract requirements until completion of work.

#### Additional Preparatory and Initial Phases:  Additional preparatory and initial phases may be required on the same feature of work for the following reasons:

##### Quality of on-going work is unacceptable.

##### Changes occur in the applicable quality control staff, on-site production supervision, or work crew.

##### Work on a particular feature of work is resumed after a substantial period of inactivity.

## DOCUMENTATION

#### Maintain Quality Control Daily Reports and Daily Test Report Information Sheets (samples attached) of quality control activities and tests.

#### Quality Control Daily Reports may not be substituted for other written reports required under clauses of the contract, such as Disputes, Differing Site Conditions, or Changes.

## ENFORCEMENT

#### The Contractor shall stop work on any item or feature pending satisfactory correction of any deficiency noted by the quality control staff or the Contracting Officer.

END OF SECTION 01430

#### CONTRACTOR’S QUALITY CONTROL DAILY REPORT

#### REPORT NO.                                            SHEET 1 OF

| PROJECT |  |  |  |  |  | CONTRACT NO. |  |  |  |  |  |  | DATE |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FOREST |  |  |  |  |  | CONTRACTOR'S REPRESENTATIVE ON THE JOB |  |  |  |  |  |  |  |  |  |  |
| WEATHER (Rain, Snow, Cloudy, Windy, etc.) | RAINFALL Inches | TEMPERATURE |  |  |  | GROUND CONDITIONS (Dry, Damp, Wet, Frozen, etc.) |  |  |  |  |  |  |  |  |  |  |
|  |  | MAX. |  | MIN. |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. PRIME CONTRACTOR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NO. EMPLOYEES BY JOB CATEGORIES |  | Hours |  | HEAVY EQUIPMENT ON JOB |  |  |  | NO. UNITS |  |  | HRS. WORKING |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | Comments |  |  |
| WORK PERFORMED BY PRIME CONTRACTOR: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MATERIALS DELIVERED |  |  |  |  |  | OFFICIAL VISITORS TO SITE |  |  |  |  |  |  |  |  |  |  |
| 2A. SUBCONTRACTOR, _____________________________________________: (If more than one subcontractor use copies of following page.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NO. EMPLOYEES BY JOB CATEGORIES |  | Hours |  | HEAVY EQUIPMENT ON JOB |  |  |  | NO. UNITS |  |  | HRS. WORKING |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | Comments |  |  |
| WORK PERFORMED BY SUBCONTRACTOR: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. SPECIFIC INSPECTIONS: (Inspections performed, results, and corrective actions) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. TESTING: 1 Check if any testing was performed today. (Complete and attach Test Report Information Sheets.) Type and Location of Testing: ____________________________________________________________________________________ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. VERBAL INSTRUCTION RECEIVED FROM GOVERNMENT ON CONSTRUCTION DEFICIENCIES OR RE-TESTING REQUIRED: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6. REMARKS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7. CERTIFICATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I certify that the above report is complete and correct and that I, or my authorized representative, have inspected all work performed this day by the prime contractor and each subcontractor and determined that all materials, equipment, and workmanship are in strict compliance with the plans and specifications except as may be noted above. ____________________________________________________________ Contractor's Quality Control Representative |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SUBCONTRACTOR WORK CONTINUED: |  |  |  |  |  |  | CONTRACT NO. |  |  | REPORT NO. _______ SHEET ____ OF ____ |  |  |  |  |  |  |
| 2 SUBCONTRACTOR, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NO. EMPLOYEES BY JOB CATEGORIES |  |  | Hours |  | HEAVY EQUIPMENT ON JOB |  |  |  | NO. UNITS | HRS. WORKING |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | YES |  | NO |  |  | Comments |  |
| WORK PERFORMED BY SUBCONTRACTOR: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 SUBCONTRACTOR, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NO. EMPLOYEES BY JOB CATEGORIES |  |  | Hours |  | HEAVY EQUIPMENT ON JOB |  |  |  | NO. UNITS | HRS. WORKING |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | YES |  | NO |  |  | Comments |  |
| WORK PERFORMED BY SUBCONTRACTOR: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 SUBCONTRACTOR, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NO. EMPLOYEES BY JOB CATEGORIES |  |  | Hours |  | HEAVY EQUIPMENT ON JOB |  |  |  | NO. UNITS | HRS. WORKING |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | YES |  | NO |  |  | Comments |  |
| WORK PERFORMED BY SUBCONTRACTOR: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 SUBCONTRACTOR, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NO. EMPLOYEES BY JOB CATEGORIES |  |  | Hours |  | HEAVY EQUIPMENT ON JOB |  |  |  | NO. UNITS | HRS. WORKING |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | YES |  | NO |  |  | COMMENTS |  |
| WORK PERFORMED BY SUBCONTRACTOR: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

DAILY TEST REPORT INFORMATION SHEET

CONTRACT NO.                              REPORT NO. _____________

SHEET _____ OF _________

| 1. Individual Making Inspection or Test: |  |  |
| --- | --- | --- |
| 2. Testing Laboratory; Name: | Phone #: |  |
| Address: |  |  |
| 3. Description of Work and Test Method: |  |  |
| 4. Location of Samples and Tests or Inspections: |  |  |
| 5. Specification Section: |  |  |
| 6. Inspection or Test Data: |  |  |
| 7. Test Results and Interpretations of Test Results: |  |  |
| 8. Comments or Professional Opinion About Compliance of Inspected Work or Tested Work with contract Document Requirements: |  |  |
| 9. Recommendations: |  |  |
| 10. Corrective Actions Taken: |  |  |
| CERTIFICATION: I certify that the above testing report is complete and correct and that all testing performed this day for this contract is in strict compliance with the plans and specifications except as noted above. |  |  |
|  |  | Signature of Inspector |