SECTION 05510

METAL STAIRS

- GENERAL

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - This Section includes the following:

          - Industrial-type stairs with steel grating treads.

          - Steel tube railings attached to metal stairs.

          - Railing gates at the level of exit discharge.

        - Related Sections include the following:

          - Division 05 Section "Metal Fabrications" for metal treads and nosings not installed in metal stairs.

          - Division 05 Section "Pipe and Tube Railings" for pipe and tube railings not attached to metal stairs.

      - PERFORMANCE REQUIREMENTS

        - Structural Performance of Stairs:  Provide metal stairs capable of withstanding the effects of gravity loads and the following loads and stresses within limits and under conditions indicated:

          - Uniform Load:  100 lbf/sq. ft. (4.79 kN/sq. m).

          - Concentrated Load:  1000 lbf (1.33 kN) applied on an area of 4 sq. in. (2580 sq. mm).

          - Uniform and concentrated loads need not be assumed to act concurrently.

          - Stair Framing:  Capable of withstanding stresses resulting from railing loads in addition to loads specified above.

          - Limit deflection of treads, platforms, and framing members to L/240 or 1/4 inch (6.4 mm), whichever is less.

        - Structural Performance of Railings:  Provide railings capable of withstanding the effects of gravity loads and the following loads and stresses within limits and under conditions indicated:

          - Handrails:

            - Concentrated load of 200 lbf (0.89 kN) applied in any direction.

          - Top Rails of Guards:

            - Concentrated load of 200 lbf (0.89 kN) applied in any direction.

          - Infill of Guards:

            - Uniform load of 25 lbf/sq. ft. (1.2 kN/sq. m) applied horizontally.

            - Infill load and other loads need not be assumed to act concurrently.

        - Seismic Performance:  Provide metal stairs capable of withstanding the effects of earthquake motions determined according to ASCE 7, "Minimum Design Loads for Buildings and Other Structures":  Section 9, "Earthquake Loads."

      - SUBMITTALS

        - Product Data:  For metal stairs and the following:

          - Paint products.

          - Grout.

        - Shop Drawings:  Include plans, elevations, sections, details, and attachments to other work.

          - Provide templates for anchors and bolts specified for installation under other Sections.

          - For installed products indicated to comply with design loads, include structural analysis data signed and sealed by the qualified professional engineer responsible for their preparation.

        - Welding certificates.

        - Qualification Data:  For professional engineer.

        - Product Test Reports:  Based on evaluation of comprehensive tests performed by a qualified testing agency, for stairs and railings.

          - Test railings according ASTM E 894 and ASTM E 935.

      - QUALITY ASSURANCE

        - Installer Qualifications:  Fabricator of products.

        - NAAMM Stair Standard:  Comply with "Recommended Voluntary Minimum Standards for Fixed Metal Stairs" in NAAMM AMP 510, "Metal Stairs Manual," for class of stair designated, unless more stringent requirements are indicated.

          - Industrial-Type Stairs:  Industrial class.

        - Welding:  Qualify procedures and personnel according to the following:

          - AWS D1.1, "Structural Welding Code--Steel."

      - COORDINATION

        - Coordinate installation of anchorages for metal stairs.  Furnish setting drawings, templates, and directions for installing anchorages, including sleeves, concrete inserts, anchor bolts, and items with integral anchors, that are to be embedded in concrete.  Deliver such items to Project site in time for installation.

        - Coordinate locations of hanger rods and struts with other work so that they will not encroach on required stair width.

- PRODUCTS

      - MANUFACTURERS

        - In other Part 2 articles where titles below introduce lists, the following requirements apply to product selection:

          - Available Products:  Subject to compliance with requirements, products that may be incorporated into the Work include, but are not limited to, products specified.

          - Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, manufacturers specified.

      - METALS, GENERAL

        - Metal Surfaces, General:  Provide materials with smooth, flat surfaces, unless otherwise indicated.  For components exposed to view in the completed Work, provide materials without seam marks, roller marks, rolled trade names, or blemishes.

      - FERROUS METALS

        - Steel Plates, Shapes, and Bars:  ASTM A 588/A 588M.

        - Steel Tubing:  ASTM A847.

        - Steel Bars for Grating Treads:  ASTM A 36/A 36M.

        - Wire Rod for Grating Crossbars:  ASTM A 510 (ASTM A 510M).

      - FASTENERS

        - General:  Provide corrosion resistant steel fasteners.

        - Bolts and Nuts:  Heavy hexagon-head bolts, ASTM A 325, Type 3 (ASTM F 568M, Property Class 4.6); with corrosion resistant heavy hex nuts, ASTM A 563 (ASTM A 563M); and, where indicated, flat washers.

        - Anchor Rods:  ASTM F 1554, Grade 36.

          - Provide hot-dip or mechanically deposited, zinc-coated anchor bolts for exterior stairs.

        - Machine Screws:  ASME B18.6.3 (ASME B18.6.7M).

        - Plain Washers:  Round, ASME B18.22.1 (ASME B18.22M).

        - Lock Washers:  Helical, spring type, ASME B18.21.1 (ASME B18.21.2M).

        - Expansion Anchors:  Anchor bolt and sleeve assembly with capability to sustain, without failure, a load equal to six times the load imposed when installed in unit masonry and four times the load imposed when installed in concrete, as determined by testing according to ASTM E 488, conducted by a qualified independent testing agency.

          - Material for Anchors in Exterior Locations:  Alloy Group 1 (A1) stainless-steel bolts complying with ASTM F 593 (ASTM F 738M) and nuts complying with ASTM F 594 (ASTM F 836M).

      - MISCELLANEOUS MATERIALS

        - Welding Rods and Bare Electrodes:  Select according to AWS specifications for metal alloy welded.

        - Nonshrink, Nonmetallic Grout:  Factory-packaged, nonstaining, noncorrosive, nongaseous grout complying with ASTM C 1107.  Provide grout specifically recommended by manufacturer for interior and exterior applications.

      - FABRICATION, GENERAL

        - Provide complete stair assemblies, including metal framing, hangers, struts, railings, clips, brackets, bearing plates, and other components necessary to support and anchor stairs and platforms on supporting structure.

          - Join components by welding, unless otherwise indicated.

          - Use connections that maintain structural value of joined pieces.

        - Preassembled Stairs:  Assemble stairs in shop to greatest extent possible.  Disassemble units only as necessary for shipping and handling limitations.  Clearly mark units for reassembly and coordinated installation.

        - Cut, drill, and punch metals cleanly and accurately.  Remove burrs and ease edges to a radius of approximately 1/32 inch (1 mm), unless otherwise indicated.  Remove sharp or rough areas on exposed surfaces.

        - Form bent-metal corners to smallest radius possible without causing grain separation or otherwise impairing work.

        - Form exposed work true to line and level with accurate angles and surfaces and straight edges.

        - Weld connections to comply with the following:

          - Use materials and methods that minimize distortion and develop strength and corrosion resistance of base metals.

          - Obtain fusion without undercut or overlap.

          - Remove welding flux immediately.

          - Weld exposed corners and seams continuously, unless otherwise indicated.

          - At exposed connections, finish exposed welds and surfaces smooth and blended so no roughness shows after finishing and contour of welded surface matches that of adjacent surface.

        - Form exposed connections with hairline joints, flush and smooth, using concealed fasteners where possible.  Where exposed fasteners are required, use Phillips flat-head (countersunk) screws or bolts unless otherwise indicated.  Locate joints where least conspicuous.

        - Fabricate joints that will be exposed to weather in a manner to exclude water.  Provide weep holes where water may accumulate.

      - STEEL-FRAMED STAIRS

        - Available Manufacturers:

          - Alfab, Inc.

          - American Stair, Inc.

          - Sharon Companies Ltd. (The).

        - Stair Framing:

          - Fabricate stringers of steel channels.

            - Provide closures for exposed ends of channel stringers.

          - Construct platforms of steel channel headers and miscellaneous framing members as needed to comply with performance requirements.

          - Weld or bolt stringers to headers; weld or bolt framing members to stringers and headers.

        - Metal Bar-Grating Stairs:  Form treads and platforms to configurations shown from metal bar grating; fabricate to comply with NAAMM MBG 531, "Metal Bar Grating Manual."

          - Fabricate treads and platforms from pressure-locked steel grating with 1-by-3/16-inch (25-by-5-mm) bearing bars at 11/16 inch (17 mm) o.c. and crossbars at 4 inches (100 mm) o.c., NAAMM designation:  P-11-4 (1 x 3/16) STEEL.

          - Surface:  Serrated.

          - Finish:  Galvanized.

          - Fabricate grating treads with cast abrasive nosing and with steel angle or steel plate carrier at each end for stringer connections.  Secure treads to stringers with bolts.

          - Fabricate grating platforms with nosing matching that on grating treads.  Provide toeplates at open-sided edges of grating platforms.  Weld grating to platform framing.

      - STEEL TUBE RAILINGS

        - General:  Fabricate railings to comply with requirements indicated for design, dimensions, details, finish, and member sizes, including wall thickness of tube, post spacings, and anchorage, but not less than that needed to withstand indicated loads.

          - Configuration:  1-5/8-inch- (41-mm-) diameter top, bottom, and intermediate rails and 1-1/2-inch- (38-mm-) square posts.  Space intermediate rails less than 19 inches (483 mm)] clear.

          - Gates:  Form gates from steel tube of same size and shape as top rails, with infill to match guards.  Provide with cam-type self-closing or spring hinges for fastening to supporting structure and overlapping stop with rubber bumper to prevent gate from opening in direction opposite egress.

        - Welded Connections:  Fabricate railings with welded connections.  Cope components at connections to provide close fit, or use fittings designed for this purpose.  Weld all around at connections, including at fittings.

        - Form changes in direction of railings as follows:

          - By flush bends or by inserting prefabricated flush-elbow fittings.

        - Form simple and compound curves by bending members in jigs to produce uniform curvature for each repetitive configuration required; maintain cross section of member throughout entire bend without buckling, twisting, cracking, or otherwise deforming exposed surfaces of components.

        - Close exposed ends of railing members with prefabricated end fittings.

        - Flanges, Fittings, and Anchors:  Provide end closures, flanges, miscellaneous fittings, and anchors for interconnecting components and for attaching to other work.  Furnish inserts and other anchorage devices for connecting to concrete work.

          - Connect posts to stair framing by direct welding, unless otherwise indicated.

          - For nongalvanized railings, provide nongalvanized ferrous-metal fittings, brackets, fasteners, and sleeves, except galvanize anchors embedded in concrete construction.

        - Fillers:  Provide fillers made from steel plate, or other suitably crush-resistant material, where needed to transfer loads to structural supports.  Size fillers to suit finish thicknesses and to produce adequate bearing area to prevent bracket rotation and overstressing of substrate.

      - STAIR RAILINGS

        - Comply with applicable requirements in Division 05 Section "Pipe and Tube Railings for railings, and as follows:

          - Rails may be bent at corners, rail returns, and wall returns, instead of using prefabricated fittings.

          - Connect posts to stair framing by direct welding, unless otherwise indicated.

      - FINISHES

        - Comply with NAAMM's "Metal Finishes Manual for Architectural and Metal Products" for recommendations for applying and designating finishes.

        - Galvanizing:  Hot-dip galvanize items as indicated to comply with applicable standard listed below:

          - ASTM A 153/A 153M, for galvanizing steel and iron hardware.

          - Fill vent and drain holes that will be exposed in finished Work, unless indicated to remain as weep holes, by plugging with zinc solder and filing off smooth.

- EXECUTION

      - INSTALLATION, GENERAL

        - Fastening to In-Place Construction:  Provide anchorage devices and fasteners where necessary for securing metal stairs to in-place construction.  Include threaded fasteners for concrete inserts, through-bolts and other connectors.

        - Cutting, Fitting, and Placement:  Perform cutting, drilling, and fitting required for installing metal stairs.  Set units accurately in location, alignment, and elevation, measured from established lines and levels and free of rack.

        - Install metal stairs by welding stair framing to steel structure or to weld plates cast into concrete, unless otherwise indicated.

        - Provide temporary bracing or anchors in formwork for items that are to be built into concrete or similar construction.

        - Fit exposed connections accurately together to form hairline joints.  Weld connections that are not to be left as exposed joints but cannot be shop welded because of shipping size limitations.  Do not weld, cut, or abrade surfaces of exterior units that have been hot-dip galvanized after fabrication and are for bolted or screwed field connections.

        - Field Welding:  Comply with the following requirements:

          - Use materials and methods that minimize distortion and develop strength and corrosion resistance of base metals.

          - Obtain fusion without undercut or overlap.

          - Remove welding flux immediately.

          - At exposed connections, finish exposed welds and surfaces smooth and blended so no roughness shows after finishing and contour of welded surface matches that of adjacent surface.

      - INSTALLING METAL STAIRS WITH GROUTED BASEPLATES

        - Clean concrete bearing surfaces of bond-reducing materials, and roughen to improve bond to surfaces.  Clean bottom surface of baseplates.

        - Set steel stair baseplates on wedges, shims, or leveling nuts.  After stairs have been positioned and aligned, tighten anchor bolts.  Do not remove wedges or shims but, if protruding, cut off flush with edge of bearing plate before packing with grout.

          - Use nonmetallic, nonshrink grout, unless otherwise indicated.

          - Pack grout solidly between bearing surfaces and plates to ensure that no voids remain.

      - INSTALLING STEEL TUBE RAILINGS

        - Adjust railing systems before anchoring to ensure matching alignment at abutting joints.  Space posts at spacing indicated or, if not indicated, as required by design loads.  Plumb posts in each direction.  Secure posts and rail ends to building construction as follows:

          - Anchor posts to steel by welding directly to steel supporting members.

      - ADJUSTING AND CLEANING

        - Galvanized Surfaces:  Clean field welds, bolted connections, and abraded areas and repair galvanizing to comply with ASTM A780.

END OF SECTION 055100