SECTION 10431

- signs

part 1 - GENERAL

## SUMMARY

#### This Section includes the following:

##### Panel signs.

##### Signage accessories.

## SUBMITTALS

#### Product Data:  For each product indicated.

#### Samples:  For each sign material indicated that involves color selection.

## QUALITY ASSURANCE

#### Regulatory Requirements:  Comply with the Americans with Disabilities Act (ADA) and with code provisions as adopted by authorities having jurisdiction.

part 2 - PRODUCTS

## MANUFACTURERS

#### In other Part 2 articles where titles below introduce lists, the following requirements apply for product selection:

##### Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the manufacturers specified.

## PANEL SIGNS

#### General:  Provide panel signs that comply with requirements indicated for materials, thicknesses, finishes, colors, designs, shapes, sizes, and details of construction.

#### Available Manufacturers:

##### Allenite Signs; Allen Marking Products, Inc.

##### American Graphics Inc.

##### Andco Industries Corp.

##### APCO Graphics, Inc.

##### ASI Sign Systems, Inc.

##### Best Manufacturing Co.

##### Grimco, Inc.

##### Innerface Sign Systems, Inc.

##### Kaltech Industries Group, Inc.

##### Mills Manufacturing, Inc.

##### Mohawk Sign Systems.

##### Seton Identification Products.

##### Signature Signs, Inc.

##### Supersine Company (The).

#### Aluminum Sheet and Plate:  ASTM B 209 (ASTM B 209M), alloy and temper recommended by aluminum producer and finisher for type of use and finish indicated, and with not less than the strength and durability properties of 5005-H15.

#### Unframed Panel Signs:  Fabricate signs with edges mechanically and smoothly finished to comply with the following requirements:

##### Edge Condition:  Square cut.

##### Corner Condition:  Rounded to radius indicated.

#### Graphic Content and Style:  Provide sign copy that complies with requirements indicated in the Sign Schedule for size, style, spacing, content, mounting height and location, material, finishes, and colors of signage.

## ACCESSORIES

#### Mounting Methods:  Use fence brackets fabricated from materials that are not corrosive to sign material and mounting surface.

part 3 - EXECUTION

## INSTALLATION

#### General:  Locate signs and accessories where indicated, using mounting methods of types described and in compliance with manufacturer's written instructions.

##### Install signs level, plumb, and at heights indicated, with sign surfaces free from distortion and other defects in appearance.

##### Exterior Wall Signs:  Install “Authorized Personnel Only” signs on 4 sides of chain-link fence gate.   Install “Lightning Electrocution Hazard” sign on the East side of long side of the chain-link fence next to the gate.

#### Fence-Mounted Panel Signs:  Attach panel signs to chain-link fence using approved manufacturer’s method.

PART 4 – SIGN SCHEDULE

## SIGN SCHEDULE

#### Standard Aluminimum Sign – 4 required

##### OSHA Aprroved

##### Size: 14” x 10”

##### Thickness: 0.08 inch

#### Custom Aluminum Sign – 1 required

##### Size: 24” x 36”

##### Radius: 1 ½” corners

##### Letters: 1” block

##### Letter color: Black

##### Background color: Yellow

##### Thickness: 0.12 inch

LIGHTNING

ELECTROCUTION  HAZARD

This tower is never to be used

during any storm event

or potential threat of a storm.

Immediately evacuate if:

Storm clouds are visible within 10 mile radius

Thunder is heard

Lightning is seen

Remain evacuated 45 minutes after:

Storm clouds are more than 10 miles away

Last thunder is heard

Last lightning is seen

WARNING

END OF SECTION 10431