SECTION 01700

- EXECUTION REQUIREMENTS

part 1 - GENERAL

## SUMMARY

#### This Section includes general procedural requirements governing execution of the Work including, but not limited to, the following:

##### Construction layout.

##### Field engineering and surveying.

##### General installation of products.

##### Progress cleaning.

##### Starting and adjusting.

##### Protection of installed construction.

##### Correction of the Work.

#### See Division 1 Section "Closeout Procedures" for submitting final property survey with Project Record Documents, recording of Government-accepted deviations from indicated lines and levels, and final cleaning.

## SUBMITTALS

#### Certificates:  Submit certificate signed by land surveyor certifying that location and elevation of improvements comply with requirements.

#### Landfill Receipts:  Submit copy of receipts issued by a landfill facility, licensed to accept hazardous materials, for hazardous waste disposal.

#### Certified Surveys:  Submit two copies signed by land surveyor.

#### Final Property Survey:  Submit five copies showing the Work performed and record survey data.

## QUALITY ASSURANCE

#### Land Surveyor Qualifications:  A professional land surveyor who is legally qualified to practice in jurisdiction where Project is located and who is experienced in providing land-surveying services of the kind indicated.

part 2 - PRODUCTS (Not Used)

part 3 – EXECUTION

## EXAMINATION

#### Existing Conditions:  The existence and location of site improvements, utilities, and other construction indicated as existing are not guaranteed.  Before beginning work, investigate and verify the existence and location of mechanical and electrical systems and other construction affecting the Work.

##### Before construction, verify the location and points of connection of utility services.

#### Existing Utilities:  The existence and location of underground and other utilities and construction indicated as existing are not guaranteed.  Before beginning sitework, investigate and verify the existence and location of underground utilities and other construction affecting the Work.

##### Furnish location data for work related to Project that must be performed by public utilities serving Project site.

#### Acceptance of Conditions:  Examine substrates, areas, and conditions, with Installer or Applicator present where indicated, for compliance with requirements for installation tolerances and other conditions affecting performance.  Record observations.

##### Verify compatibility with and suitability of substrates, including compatibility with existing finishes or primers.

##### Examine roughing-in for mechanical and electrical systems to verify actual locations of connections before equipment and fixture installation.

##### Proceed with installation only after unsatisfactory conditions have been corrected.  Proceeding with the Work indicates acceptance of surfaces and conditions.

## PREPARATION

#### Existing Utility Information:  Furnish information to Contracting Officer (CO) that is necessary to adjust, move, or relocate existing utility structures, utility poles, lines, services, or other utility appurtenances located in or affected by construction.  Coordinate with authorities having jurisdiction.

#### Existing Utility Interruptions:  Do not interrupt utilities serving facilities occupied by Owner or others unless permitted under the following conditions and then only after arranging to provide temporary utility services according to requirements indicated:

##### Notify Contracting Officer (CO) not less than two days in advance have proposed utility interruptions.

##### Do not proceed with utility interruptions without CO’s written permission.

#### Field Measurements:  Take field measurements as required to fit the Work properly.  Recheck measurements before installing each product.  Where portions of the Work are indicated to fit to other construction, verify dimensions of other construction by field measurements before fabrication.  Coordinate fabrication schedule with construction progress to avoid delaying the Work.

#### Space Requirements:  Verify space requirements and dimensions of items shown diagrammatically on Drawings.

#### Review of Contract Documents and Field Conditions:  Immediately on discovery of the need for clarification of the Contract Documents, submit a request for information to CO.  Include a detailed description of problem encountered, together with recommendations for changing the Contract Documents.

## CONSTRUCTION LAYOUT

#### Verification:  Before proceeding to lay out the Work, verify layout information shown on Drawings, in relation to the property survey and existing benchmarks.  If discrepancies are discovered, notify CO promptly.

#### General:  Engage a land surveyor to lay out the Work using accepted surveying practices.

##### Establish benchmarks and control points to set lines and levels at each story of construction and elsewhere as needed to locate each element of Project.

##### Establish dimensions within tolerances indicated.  Do not scale Drawings to obtain required dimensions.

##### Inform installers of lines and levels to which they must comply.

##### Check the location, level and plumb, of every major element as the Work progresses.

##### Notify CO when deviations from required lines and levels exceed allowable tolerances.

##### Close site surveys with an error of closure equal to or less than the standard established by authorities having jurisdiction.

#### Site Improvements:  Locate and lay out site improvements, including pavements, grading, fill and topsoil placement, utility slopes, and invert elevations.

#### Building Lines and Levels:  Locate and lay out control lines and levels for structures, building foundations, column grids, and floor levels, including those required for mechanical and electrical work.  Transfer survey markings and elevations for use with control lines and levels.  Level foundations and piers from two or more locations.

#### Record Log:  Maintain a log of layout control work.  Record deviations from required lines and levels.  Include beginning and ending dates and times of surveys, weather conditions, name and duty of each survey party member, and types of instruments and tapes used.  Make the log available for reference by CO.

## FIELD ENGINEERING

#### Reference Points:  Locate existing permanent benchmarks, control points, and similar reference points before beginning the Work.  Preserve and protect permanent benchmarks and control points during construction operations.

#### Benchmarks:  Establish and maintain a minimum of two permanent benchmarks on Project site, referenced to data established by survey control points.  Comply with authorities having jurisdiction for type and size of benchmark.

##### Record benchmark locations, with horizontal and vertical data, on Project Record Documents.

## INSTALLATION

#### General:  Locate the Work and components of the Work accurately, in correct alignment and elevation, as indicated.

##### Make vertical work plumb and make horizontal work level.

##### Where space is limited, install components to maximize space available for maintenance and ease of removal for replacement.

##### Conceal pipes, ducts, and wiring in finished areas, unless otherwise indicated.

#### Comply with manufacturer's written instructions and recommendations for installing products in applications indicated.

#### Install products at the time and under conditions that will ensure the best possible results.  Maintain conditions required for product performance until Substantial Completion.

#### Conduct construction operations so no part of the Work is subjected to damaging operations or loading in excess of that expected during normal conditions of occupancy.

#### Anchors and Fasteners:  Provide anchors and fasteners as required to anchor each component securely in place, accurately located and aligned with other portions of the Work.

##### Mounting Heights:  Where mounting heights are not indicated, mount components at heights directed by CO.

##### Allow for building movement, including thermal expansion and contraction.

#### Joints:  Make joints of uniform width.  Where joint locations in exposed work are not indicated, arrange joints for the best visual effect.  Fit exposed connections together to form hairline joints.

#### Hazardous Materials:  Use products, cleaners, and installation materials that are not considered hazardous.

## PROGRESS CLEANING

#### General:  Clean Project site and work areas daily, including common areas.  Coordinate progress cleaning for joint-use areas where more than one installer has worked.  Enforce requirements strictly.  Dispose of materials lawfully.

##### Comply with requirements in NFPA 241 for removal of combustible waste materials and debris.

##### Do not hold materials more than 7 days during normal weather or 3 days if the temperature is expected to rise above 80 deg F (27 deg C).

##### Containerize hazardous and unsanitary waste materials separately from other waste.  Mark containers appropriately and dispose of legally, according to regulations.

#### Site:  Maintain Project site free of waste materials and debris.

#### Work Areas:  Clean areas where work is in progress to the level of cleanliness necessary for proper execution of the Work.

##### Remove liquid spills promptly.

##### Where dust would impair proper execution of the Work, broom-clean or vacuum the entire work area, as appropriate.

#### Installed Work:  Keep installed work clean.  Clean installed surfaces according to written instructions of manufacturer or fabricator of product installed, using only cleaning materials specifically recommended.  If specific cleaning materials are not recommended, use-cleaning materials that are not hazardous to health or property and that will not damage exposed surfaces.

#### Concealed Spaces:  Remove debris from concealed spaces before enclosing the space.

#### Waste Disposal:  Burying or burning waste materials on-site will not be permitted.  Washing waste materials down sewers or into waterways will not be permitted.

#### During handling and installation, clean and protect construction in progress and adjoining materials already in place.  Apply protective covering where required to ensure protection from damage or deterioration at Substantial Completion.

#### Clean and provide maintenance on completed construction as frequently as necessary through the remainder of the construction period.  Adjust and lubricate operable components to ensure operability without damaging effects.

#### Limiting Exposures:  Supervise construction operations to assure that no part of the construction, completed or in progress is subject to harmful, dangerous, damaging, or otherwise deleterious exposure during the construction period.

## STARTING AND ADJUSTING

#### Start equipment and operating components to confirm proper operation.  Remove malfunctioning units, replace with new units, and retest.

#### Adjust operating components for proper operation without binding.  Adjust equipment for proper operation.

#### Test each piece of equipment to verify proper operation.  Test and adjust controls and safeties.  Replace damaged and malfunctioning controls and equipment.

## PROTECTION OF INSTALLED CONSTRUCTION

#### Provide final protection and maintain conditions that ensure installed Work is without damage or deterioration at time of Substantial Completion.

#### Comply with manufacturers written instructions for temperature and relative humidity.

## CORRECTION OF THE WORK

#### Repair or remove and replace defective construction.  Restore damaged substrates and finishes.

##### Repairing includes replacing defective parts, refinishing damaged surfaces, touching up with matching materials, and properly adjusting operating equipment.

#### Restore permanent facilities used during construction to their specified condition.

#### Remove and replace damaged surfaces that are exposed to view if surfaces cannot be repaired without visible evidence of repair.

#### Repair components that do not operate properly.  Remove and replace operating components that cannot be repaired.

#### Remove and replace chipped, scratched, and broken glass or reflective surfaces.

END OF SECTION 01700