SECTION 01010

- SUMMARY of work

PART 1 - GENERAL

## summary

#### Work for this project consists of excavating and backfilling for footings, supplying and constructing reinforced concrete footings, supplying and erecting a steel rappel tower and simulator, including stairs, handrailing system, rappelling skids, chain link fencing and all other items shown on the drawings.

## location

#### This project is located at Aerial Fire Depot adjacent to the Missoula International Airport in Missoula, MT.

## GENERAL SITE CONDITIONS AND WEATHER

#### The elevation of the site is approximately 3,200 feet above mean sea level.  The construction site experiences moderate snowfall and moderate to cold temperatures, with snow on the ground typically from December through April.

#### In-depth soil surveys have been performed and the geotechnical report is available upon request.  The Contractor should expect to working in silty gravel mixed with sand and clay.

#### The water table was not encountered during the geotechnical investigation; however, if water is encountered during excavation, the Contractor shall perform adequate dewatering to keep the excavation free from water and dispose of the water without damage to property.

## USE OF PREMISES

#### General:  Contractor shall have limited use of premises for construction operations, including use of Project site, during construction period.  Contractor's use of premises is limited by the Government's right to perform work at the Aerial Fire Depot to provide Wildfire Protection Activities.

#### Contractor shall at all times conduct his operations to ensure the least inconvenience to the public and employees.

#### Confine storage of materials to areas as approved by the CO.

#### Contractor shall provide adequate signing and barricades and take necessary safety measures to protect the public during all construction operations.  Contractor shall minimize disturbance of all undisturbed areas.

#### Preservation of Natural Features:  Confines all operations to work limits of the project.  Prevent damage to natural surroundings.  Restore damaged areas, repairing or replacing damaged trees and plants, at no additional expense to the Government.

##### Provide temporary barriers to protect existing trees and plants and root zones.

##### Do not remove, injure, or destroy trees or other plants without prior approval.  Consult with CO and remove agreed-on roots and branches that interfere with construction.

##### Do not fasten ropes, cables, or guys to existing trees.

##### Carefully supervise excavating, grading, filling, and other construction operations near trees to prevent damage.

#### Existing Utilities:  Notify CO and utility companies of proposed locations and times for excavation.

##### Contractor shall be responsible for locating and preventing damage to known utilities.  If damage occurs, repair utility at no additional expense to the Government.

##### Interruption of Existing Utility Service:  Notify CO, 48 hours in advance of any interruption of existing utilities.

#### Airport and Aerial Fire Depot Notification: Contractor will be responsible to notify the Missoula International Airport and Aerial Fire Depot of any construction activities that may effect operations at the airport.

## MEALS AND LODGING

#### Meals and lodging are commercially available in the City of Missoula within 5 miles of the construction site.  Meals or lodging will not be available at Forest Service facilities.

## camping

#### Camping is not allowed at the construction site.

## construction sequence

#### There is no specified construction sequence or season for this project.

#### Depending on conditions, normal fire restrictions may apply to activities associated with this project.

#### The Contractor may have an option to develop water and/or power utilities in the early stages of construction so they may be used for the remainder of the construction project.

## Field verification

#### Field verify all new and existing dimensions affecting the work of this contract before ordering products.

## permits

#### Special permits for electrical work or any other permits required by County, State, or Federal laws or regulations shall be obtained by the Contractor.

PART 2 - PRODUCTS (Not Used)

PART 3 - EXECUTION (Not Used)

END OF SECTION 01100