RAPPEL TOWER

INITIAL INSPECTION

CHECKLIST FORM

(To be completed by Base Manager and Regional Bridge Engineer)

| Tower Name | Missoula AFD Rappel Tower |
| --- | --- |
| Date of Inspection |  |
| Inspected By |  |

|  | Yes | No | Comments |
| --- | --- | --- | --- |
| Tower |  |  |  |
| Has the Regional Bridge Engineer or a qualified structural engineer inspected the tower before initial use? |  |  |  |
| Did they find any problems or concerns that needed to be fixed? |  |  |  |
| Have all these problems or concerns been fixed or addressed? |  |  |  |
| Have they given written permission that the tower may be used for training? |  |  |  |
| Have they provided the Helibase with an initial inspection report for your records? |  |  |  |
| Are all faying surfaces at all connections in firm contact? |  |  |  |
| Is maximum thickness for fillers at faying surface less then ¼”? |  |  |  |
| Are all anchor rods and bolts snug and tightened to 200 ft-lbs torque? |  |  |  |
| Is at least one full thread for all bolts and anchor rods projected beyond the face of the nut? |  |  |  |
| Are “X” bracing rods installed under tension with no detectable sag? |  |  |  |
| Do all welds have a finished appearance with slag removed and no presence of undercutting, etc.? |  |  |  |
| Are all members constructed from unspliced lengths? |  |  |  |
| Are stairs installed with proper rise and run with tolerances within accepted standards? |  |  |  |
| Have all the handrails been inspected? |  |  |  |
| Are all grating properly installed with adequate attachment to supporting framework? |  |  |  |

| Do all gates have properly working closing mechanisms and latches? |  |  |  |
| --- | --- | --- | --- |
| Were all members installed square, true and plum within AISC Code of Standard Practice tolerances? |  |  |  |
| Was leveling grout installed at all column base plates to provide firm contact with bearing surfaces? |  |  |  |
| Have all sharp edges, burrs or other aspects of members been removed or mitigate to eliminate safety hazards? |  |  |  |
| Have the location of all anchor points for rappellers and spotters been verified and hardware been installed per manufacturer’s recommendations? |  |  |  |
| Simulator |  |  |  |
| Are all faying surfaces at all connections in firm contact? |  |  |  |
| Is at least one full thread for all bolts and anchor rods projected beyond the face of the nut? |  |  |  |
| Are all anchor rods and bolts snug and tightened to 200 ft-lbs torque? |  |  |  |
| Do all welds have a finished appearance with slag removed and no presence of undercutting, and completed to the extent to preclude water migration into joints etc.? |  |  |  |
| Are all members constructed from unspliced lengths? |  |  |  |
| Are all gates and doors in proper working order and have properly working closing mechanisms and latches? |  |  |  |
| Were weep holes installed and clear from obstruction? |  |  |  |
| Were all members installed square, true and plum within AISC Code of Standard Practice tolerances? |  |  |  |
| Have all sharp edges, burrs or other aspects of members been removed or mitigate to eliminate safety hazards or cause wearing of rappelling ropes? |  |  |  |
| Are all grating properly installed with adequate attachment to supporting framework? |  |  |  |

| Have the location of all anchor points for rappellers and spotters been verified and hardware been installed per manufacturer’s recommendations? |  |  |  |
| --- | --- | --- | --- |
| Is paint system undamaged or have repairs been approved? |  |  |  |