RAPPEL TOWER

DAILY PRE-USE CONDITION ASSESSMENT

CHECKLIST FORM

(To be completed by Base Manager)

| Tower Name | Missoula AFD Rappel Tower |
| --- | --- |
| Date of Inspection |  |
| Inspected By |  |

| Conditions | Yes | No | Remarks |
| --- | --- | --- | --- |
| The tower and simulator will require a daily condition assessment for the overall appearance of the tower. At a minimum, it should consist of the following questions: |  |  |  |
| Is the tower or simulator leaning or twisted? |  |  | If the tower or simulator is leaning or twisted, it should not be used and engineering should be contacted immediately. |
| Are there any broken or hanging members? |  |  | Any broken or hanging member will be required to be fixed before the tower can be used. Fixing major members may require a special inspection by the Regional Bridge Engineer or a qualified representative. |
| Are there any obvious missing parts? |  |  | Any missing parts will have to be replaced before the tower can be used. |
| Are all “X” bracing rods straight? |  |  | If any rod is bent or curved, this is an indication of a problem and the tower should not be used until the reason for the bent rod is found and corrected. |
| Have the tower and simulator been assessed for rough edges, burrs or other aspects that may cause damage to ropes before use? |  |  | All rough edges, burrs or other aspects that may cause damage to ropes and equipment must be removed or mitigated before the tower can be used. |
| Is the landing area free of obstructions and hazards? |  |  | The landing area must be free of obstructions and hazards before the tower can be used. |
| Has the landing area been loosened up prior to use? |  |  | No rappelling will be allowed if the rappel landing area is too hard and may cause knee and ankle injuries. |
| Is lightning protection system intact and functional? |  |  | Lightning protection system must be functional before the tower can be used. THE TOWER CANNOT BE USED DURING ANY KIND OF STORM. |
| Is the aircraft warning light system working? |  |  | The warning light system must function at all times and the tower may be not used when they are not functional. |
| The tower will require a daily hands-on inspection by the Base Manager. At a minimum, it should consist of the following questions: |  |  |  |
| Are stairs, walkways and landings clear of snow, ice and debris and in good condition? |  |  | All snow, ice and debris must be removed before any training is allowed on the tower. |
| Are all landings and tower decks free of trip/slip hazards (e.g., water, protruding bolts)? |  |  | All hazards will have to be removed or mitigated before the tower can be used. |
| Is the railing system complete and in good condition? |  |  | Any missing parts will have to be replaced before the tower can be used. |
| Are toe boards installed in all areas where personnel could pass underneath? |  |  | Any missing parts will have to be replaced before the tower can be used. |
| Are all areas that pose a tripping or head hazard marked in yellow? |  |  | All areas that pose a tripping or head hazard must be marked with yellow paint or tape before the tower can be used. |
| Are the access control gates and latches present, functional and in good working condition? |  |  | All gates must work and provide protect from entering areas where falling can occur before the tower can be used. |
| Have the rope anchors been checked for distortion and tightness of bolts? |  |  | This can be done by tapping on the bolts with a carabineer and if the bolts are loose, they will rattle or can be checked with a torque wrench. |
| Have all required anchors for rappelers and spotters been installed? |  |  | The lower platform should be setup before rappelers are allowed on the lower platform. |
| Is the tower plumb and free from twisting or racking? |  |  | If the tower is leaning or twisted, it should not be used and engineering should be contacted immediately. |
| Are all members in good repair - check for missing, cracked or broken parts? |  |  | Any broken or hanging member will be required to be fixed before the tower can be used. Fixing major members may require a special inspection by the Regional Bridge Engineer or a qualified representative. |
| Are all grating properly attached to supporting framework? |  |  | Any lose grating must be secured to the supporting framework before the tower can be used. |
| Have the past day’s use been reviewed and any high wind speeds, seismic events, falls during training and any other unusual events been noted? |  |  | The previous day’s paper work must be reviewed. Any problems from the day before must be fixed before the tower can be used. |
| The simulator will require a daily hands-on inspection by the Base Manager. At a minimum, it should consist of the following questions: |  |  |  |
| Is the simulator clear of snow, ice and debris and in good condition? |  |  | All snow, ice and debris should be removed before any training is allowed on the tower. |
| Is the railing system complete and in good condition? |  |  | Any missing parts will have to be replaced before the tower can be used. |
| Are all grating properly attached to supporting framework? |  |  | Any lose grating must be secured to the supporting framework before the tower can be used. |
| Are the access control gates and latches present, functional and in good working condition? |  |  | The gates must control access to the simulator; if they do function correctly the tower should not be used until fixed. |
| Have the rope anchors been checked for distortion and tightness of bolts? |  |  | This can be done by tapping on the bolts with a carabineer and if they are loose, they will rattle or checked with a torque wrench. |
| Have the simulator doors and latches been checked that they function correctly and are in good working condition? |  |  | The doors must slide easily, latch and provide safety when closed. |
| Have all required anchors for rappelers and spotters been installed? |  |  | The simulator should be setup before rappelers are allowed in the simulator. |
| Have the long and short plates, skid and J-step inspected for signs of distortion? |  |  | The plates may not be used if they show and signs of distress until they have been inspected by a qualified engineer. |
| Other Items Noted: |  |  |  |

If any of the questions have been answered with a “No”, the problem must be fixed before any training is allowed on the tower.